Residential Accessory / Building Notes

The homeowner is responsible for locating the property pins prior to setting of footings and inspection.

Structures with on grade slab foundations are limited to accessory structures on lots zoned     R (residential) or AG (agricultural).

Structures with slab on grade foundations shall not be attached at any point to the structures with frost footings.

Structures with slab on grade foundations are limited to 10’ side walls and 16’ in height, ie. At mid span between eave and peak on gable end roof.

Post- frame type building is allowed for accessory structure. The exterior shall conform to the style/décor of the principal structure or neighborhood. No corrugated or sheet metal, galvalume color shall be used for siding the dwelling or accessory building. (example; should have facia/soffit, lap siding or similar etc.) Roofing can be metal.

Accessory structures or sheds are not permitted in front yard setbacks.

Front yard setback is 25’ from property line.

Existing front yard setback less than 25’ may be matched by accessory structure, provided that overhead doors are setback at least 20’ from front property setback line.

Garage overhead doors facing rear alley must be set back 20’ from rear lot line.

Eaves on a house and garage may project 25% into the front and side yard setback. If a house setback is at 25', Unroofed porches, decks, landings, etc. may project 12' into front setback. In cases where existing house is closer than 25' to setback line, unroofed porches, decks, landings may project up to 50% of the front setback.

Walls must be at least 3' min. (or more for the inclusion of soffit) from side lot line, and 3’ min. from rear lot line and along the alleys. Any overhangs, eaves, soffits must be no closer than three (3) feet from the lot line.

No accessory building can be located in any easement on the lot.

In all residential zones, no more than forty (40) percent of lot can be developed.

Accessory structures must be at least 6’ from the principal structure.

No structure is to be built over any utility or gas line. Confirm with Utility Co.

Call 811 before digging

Last Updated 01/11/2023